Twine Tango Main Board Testing

Rev1.2

Revision History

| Version | Date | Description |
| --- | --- | --- |
| 1.0 | 18/7/2018 | Initial preliminary version |
| 1.1 | 05/08/2018 | Add Twine Logo. Update pictures |

# Introduction

## Scope

- Twine Tango Main PCB – hence “DUT”

- DUT external connectivity testing

## Purpose of Document

- Definition of Test Procedure.

- Documentation of how to perform all Main board test.

## Document Outline

Section ‎2 Tango HW Test Plan .

# Given

DUT will be tested in Tango printer with relevant assemblies. Hence not all connections will be tested.

# Actions required by Tester during Testing

Motors

Observe that motors are moving when required.

Limit Switches

- Use an object to “cut” the light beam and observe result in Twine StubsGUI .

Encoders

- Turn/move mechanics attach to encoder and observe result in Twine StubsGUI.

# Prerequisites

## Items required

- Tango printer with all relevant assemblies. Assemblies shall be in a tested/working state

- Relevant cable interconnects between assemblies and electrical cabinet

- Electrical cabinet in a tested/working state

- USB type A to type B cable

- Machine Studio Twine Stubs

- Scripts

- “Tango_Main_PCB_ATP_SN_xxx.xlsx” File for result.

# Test Procedure

## Initial steps

Record DUT serial numbers, date and time in Excel sheet. (see picture below)

- Install DUT into electrical cabinet.

- Power on Tango printer (see picture below)

Check if Status led on. If not press pushbutton switch on Cabinet

Connect USB cable between DUT and PC

Start the Tango StubsGUI and connect to DUT on 115200 and USB port that recognize.

## Read FPGAs Version.

Open “D:\ATP_Test\Scripts\Read_Version.cs”

Run script

In line xy change the filename to match the DUT serial number

Write DUT serial number on Excel (“F2” location).

Execute the script and write s/w and FPGAs version into Excel sheet(F4 Location)

## Read S/W Version.

Open “D:\ATP_Test\Scripts\read embedded version.cs

Run script

Execute the script and write s/w version into Excel sheet(F5 Location)

## Check Motors on RTFU.

Open “D:\ATP_Test\Scripts\Assemblies\RTFU\RTFU_Motors.cs”

Run script

Check if all three motors rotates.

Record results into Excel sheet (F6 Location)

## Check Limit Switch on RTFU.

Open “D:\ATP_Test\Scripts\Assemblies\RTFU\RTFU_LimitSwitch1.cs”

Run script

Activate each 5 limit switch once by inserting object which block light (see limit switch example)

Stop script when all limit switches change have been checked

Record results into Excel sheet (F7 Location)

## Check Dancer Arm and Speed Sensor on RTFU.

Open “D:\ATP_Test\Scripts\Assemblies\RTFU\RTFU_SSI.cs”

Run script

Move Dancer Arm and Speed Sensor

Observe that register values changes.

Stop script when both encoders have been checked.

Record results into Excel sheet(F8 Location)

## Check Motors on LTFU.

Open “D:\ATP_Test\Scripts\Assemblies\LTFU\LTFU_Motors.cs”

Run script

Check if all four motors rotates.

Write Result into Excel sheet (F9 Location).

## Check Limit Switch on LTFU.

Open “D:\ATP_Test\Scripts\Assemblies\LTFU\LTFU_LimitSwitch.cs”

Run script

Activate each 6 limit switch once by inserting object which block light.

Stop script when all limit switches change have been checked.

Record results into Excel sheet (F10 Location).

## Check Dancer Arm on LTFU.

Open “D:\ATP_Test\Scripts\Assemblies\LTFU\LTFU_SSI.cs”

Run script

Move each dancer arm

Observe that register values changes

Stop script when both encoders have been checked

Record results into Excel sheet (F11 Location).

## Check Motors on Winder.

Open “D:\ATP_Test\Scripts\Assemblies\Winder\Winder_Screw_Motors.cs”

Run script

Check if all two motors rotates.

Write Result into Excel sheet (“F12” location).

## Check Limit Switch on Winder.

Open “D:\ATP_Test\Scripts\Assemblies\winder\winder_LimitSwitch.cs”

Run script

Activate each 4 limit switch once by inserting object which block light.

Stop script when all limit switches change have been checked.

Record results into Excel sheet (“F13” location).

## Check Motors on Dryer.

Open “D:\ATP_Test\Scripts\Assemblies\Dryer\Dryer_Motors.cs

Run script

Check if all two motors rotates.

Write Result into Excel (“F14” location).

## Check Limit Switch on Dryer.

Open “D:\ATP_TEST\Scripts\Assemblies\Dryer\Dryer_LimitSwitch.cs”

Run script

Activate each 2 limit switch once by inserting object which block light.

Stop script when all limit switches change have been checked.

Record results into Excel sheet (“F15” location).

## Check SSRs on Dryer.

Open “D:\ATP_Test\Scripts\Assemblies\Dryer\Dryer_SSR.cs”

Run script

Check if 3 Leds turn on and off one by one.

Write Result into Excel sheet (“F16” location).

## Check Midtank_Valves.

Open “D:\ATP_Test\Scripts\Assemblies\MidTank\Midtank_Valves.cs

Run script at least 8 times, once pr. Midtank.

Enter Midtank number to test (1-8)

Observe, by listening and feeling, that valve is moving back and forth twice.

Record results into Excel sheet (“F17” location).

## Check Midtank_pressure_sensors.

Open “D:\ATP_Test\Scripts\Assemblies\MidTank\Midtank_pressure_sensors.cs

Connect connectors with resistors on it ,to J0172 and J0282 .

- The connector must connect to 5V power supplay.

Run script

Observe ADC value reported by script

- Record result into Excel sheet.it should be between two values.

## Check Dispenser 3WValve.

Open “D:\ATP_Test\Scripts\Assemblies\Dispenser\Dispenser_3WValve.cs

Run script at least 8 times, once pr. Dispenser.

Enter dispenser number to test (1-8).

Observe, by listening, that valve is moving back and forth.

Record results into Excel sheet (“E19” location).

## Check Limit Switch on Dispenser.

Open “D:\ATP_Test\Scripts\Assemblies\Dispenser\Dispenser_LimitSwitch.cs

Run script at least 8 times, once pr. Dispenser

Enter dispenser number to test (1-8):

Activate each 4 limit switch once by inserting object which block light.

Stop script when all limit switches change have been checked.

Record results into Excel sheet .

## Check Motors on Dispenser.

Open “D:\ATP_Test\Scripts\Assemblies\Dispenser\Dispenser_Motors.cs

Run script at least 8 times, once pr. Dispenser

Observe  Dispenser motor is moving back and forth

Record Result into Excel sheet .

## Check Pressure Sensor on Dispenser.

Open “D:\ATP_Test\Scripts\Assemblies\Dispenser\Dispenser_PressureSensor.cs

Run script.

Observe ADC value reported by script

Record Result into Excel sheet. It’s should be 514mv.

## Check ElectricalEnclosure_Fans.

Open “D:\ATP_Test\Scripts\Assemblies\ElectricalEnclosure\ElectricalEnclosure_Fans.cs

Run script.

Observe Tacho value reported by script

Record Result into Excel sheet Only Tacho 1 should be #04C4 .all other should be #0000

## Check PT100 (temperature sensor )

Open “D:\ATP_Test\Scripts\Assemblies\Other\PT100__Horizontal_Printing.cs”

Run script.

Observe 12  value temperature  by script. Its should be around 24 Degree

Record Result into Excel sheet.